CONTROL SYSTEMS LABORATORY

(Part I)

DEPARTMENT OF ELECTRICAL ENGINEERING

ASSAM ENGINEERING COLLEGE

JALUKBARI, GUWAHATI, ASSAM, INDIA

PIN:781013

Introduction

This laboratory course is intended to enhance the learning experience of the student in topics encountered in the subject Control Systems. This course is intended to provide assistance in solving computational problems associated with the study and application of linear control systems using MATLAB/SCI Lab or any other programming language. The first objective is to introduce the user to some of the capabilities of the softwares, so that it can be used to aid in the design and analysis of control systems. The second objective is to teach the student how to write programs and to allow them to advance rapidly in solving more advanced control system problems.

1.1 Student Responsibilities:

How the student performs in the laboratory depends on his/her preparation, participation, and teamwork. Each team member must participate in all aspects of the laboratory to ensure a thorough understanding of the computing facilities available in the laboratory.Students should emphasize on understanding the concepts and procedure for successful completion of the task.

1.2 Laboratory Teaching Assistant Responsibilities:

The Laboratory Teaching Assistant (LTA) shall provide course manualto the student during the first class.The LTA is responsible for insuring that all the necessary equipment and/orpreparations for the laboratory are available and in working condition. The LTA shall be completely familiar with each of the requirements prior toclass.The LTA should supervise the students while performing the experiment and to grade the lab notebooks and reports.

1.3 Faculty Coordinator Responsibilities:

The faculty coordinator should insure that the laboratory is properly equipped, i.e., that theteaching assistants receive any equipment necessary to complete the tasks. The coordinatoris responsible for supervising the teaching assistants and resolving any questions or problemsthat are identified by the teaching assistants or the students. The coordinator may supervise theformat of the final exam for the laboratory. He/she is also responsible for making any necessarycorrections to this manual. The faculty coordinator is responsible for ensuring that the manual is continually updated.

1.4 Laboratory Policy and Grading:

The student should understand the following policies:

Attendance

Attendance is mandatory and any absence must be for a valid excuse and mustbe documented. In case of prior knowledge of absence from class by the student, proper approval in due format must be taken from the Head of the Department. A copy of the approved leave application must be submitted to the TA/ faculty in charge of the laboratory.

Laboratory Records

The student must maintain all records while performing the experiment and he/she must prepare a report.

Grading Policy

The final grade of this course is determined according to rubrics shown in Table 1.

Table 1: Rubrics for Control Systems Lab assessment

| Category of evaluation | Components | % Allotment of Marks to the components |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0-30% | 31-60% | 61-100% |
| CONTINUOUS AND COMPREHENSIVE EVALUATION | Attendance and Preparedness (10%) | Student is present but not prepared for the laboratory classes | Student is present but not well prepared | Student is present & familiar with lab manual |
|  | Ability to do programming (Part I) and experiments (Part II) (10%) | Not able to complete the assigned work | Complete the work partially | Complete the work fully |
| RESULT & ANALYSIS | Program execution & experiment findings (10%) | Missing several important details | Missing some important details | Details are well covered |
|  | Reports (30%) | Results contain errors | Results are well presented but have some errors | Results are well presented without any errors |
| Test & Viva-voce | Familiarity with experiments and knowledge of related concepts (40%) | Able to answer at least 30% questions | Able to answer at least 50% questions | Able to answer at least 80% questions |

1.5 Course Goals and Objectives:

The laboratory course on Control Systems (Part I)is designed to provide the student with the knowledge to write programs on control system problems using MATLAB/SCI Lab software or in any other programming language. Moreexplicitly, the course objectives are:

- To gain proficiency in writing programs.

- To enhance understanding of linear control systems concepts.

- To comparemanually solved problems with computationally obtained results using the software.

1.6 Laboratory Notebooks and Reports:

The student must records and interprets his/her assignments via the laboratory notebook and thelaboratory report. The laboratory notebook is essential in recording the methodology and resultsof an experiment. Therefore, it is important to learn to keep an accurate notebook. The laboratory report is the primary means of communicating your experience and conclusionsto other professionals.

LIST OF ASSIGNMENTS

1.  Study on preliminary commands/syntax of the software

2. Study on partial fraction expansion, transfer function and

determination of transfer function from block diagrams

3.  Analysis of transient response of control system

4.  Construction of root locus and stability analysis

5.  Construction of Bode plot and Nyquist plot and stability analysis

CONTROL SYSTEMS (4th EE & IE)

COMPUTER LABORATORY

ASSIGNMENT 1

Preliminary Commands:

- Matrix and Array Operations

(a) For a square matrix A (say, a 3 x 3 matrix) test the following commands:

A^2, A’(transpose A), A(2, 3), A(2, 2), A(1,:), A(:, 1), A(:, 1:2:3), A(2:1:3, 2:1:3),

inv (A), det(A), diag(A), rank(A), expm(A), size(A), length (A), max(A), min(A)

(b) Consider two square matrices A and B and test the following operations:

A + B, A – B, A* B, A .* B, A / B, A ./ B, [A  B], [A’ B’]

- Test the following commands:

Zeros(3), zeros(3, 4); ones(5), ones(2, 3), eye(4), eye(4, 6)

- Solve for x in the equation Cx = D, where C=[1 2 1;2 3 2;-1 0 1] and D=[1; 1; 0]

- Common functions

  - Consider, x = pi/3 and find sin(x), cos(x), tan(x), exp(x), log(x), sqrt(x), round(x)

  - Consider, z = 3+4 * i (a complex number) and find abs(z), angle(z)

  - Consider, a = 0.866 and find acos(a), asin(a), atan(a)

- Consider two polynomials: p(s) = 3s2+2s+1 and q(s) = s+4, after multiplication it gives

n(s) = 3s3+14s2+9s+4. In MATLAB, this is done in following way:

>>p = [3 2 1]; q = [0 1 4];       % coefficients of polynomials

>>n = conv(p, q)

Multiply three polynomials, x(s) = s4+3s3-2s2+5s+1, y(s) = 5s2+4s-2, z(s) = 5s3-6s+7

Multiply polynomials: g(s) = (s+2)2, h(s) = s2-4s+3

- Test the following:

x = [0:1:10]

x’

p = [1:0.001:1]

p’

If x = [0   pi/2   pi   3*pi/2   2*pi] find y = sin(x)

If z = [0:pi/6:pi] find zz = cos (z) and zzz = cos(z/2)

ASSIGNMENT 2

- Partial Fraction Expansion

A rational function, , after partial fraction expansion

, pi poles, ri residues and k(s) is the direct term.

For example,  becomes

Now verify this answer using software:

In command window, type the following

>>num = [1  7  14  10  16]; den = [0  1   6  8   0]; [r, p, k] = residue (num, den) <press enter>

which gives 		r = 1, -3, 2; p = -4, -2, 0; k = 1, 1

The original function F(s) is obtained from r, p, k using the following commands

>> [num, den] = residue (r, p, k);

>>printsys (num, den, ‘s’);

Test 1: Find the partial fraction expansions of the following using MATLAB:

- Determination of zeros & poles of transfer function

Command to determine zeros (z), poles (p) and gain (k) is

[z, p, k] = tf2zp(num, den)

Where num and den are the matrices, elements of which are the coefficients of numerator and denominator polynomial respectively.

For the transfer function, , type the following in command window

>>num=[0  0  20  20]; den=[1  5  7  3]; [z,p,k]=tf2zp(num,den) <press enter>

This will produce output as z = -1, p = -3, -1, -1, k = 20

- Determination of transfer function from poles, zeros and gain

Type the following set of commands and see the results:

>> z = [-1]; p = [-3; -1; -1]; k = 20; [num, den] = zp2tf(z, p, k); printsys(num, den, ‘s’)    <press enter>

Test 2:

- Find poles, zeros and gain of all the problems given in Test 1

- For the following poles, zeros and gain, determine transfer functions:

- zeros at s = -1, -2; poles at s = 0, -4, -6 and gain k = 5

- zeros at s = 1; poles at s = 0, -2, -1+j0.5, -1-j0.5 and gain k = 10

- Series, parallel function:

Two blocks with transfer functions G1(s) and G2(s) can be multiplied (added) using the series (parallel) function:

>> sys1 = tf(num1, den1); sys2 = tf(num2, den2); sys = series (sys1, sys2) <enter>

>>sys1=tf(num1, den1); sys2 =tf(num2, den2); sys = parallel(sys1, sys2) <enter>

Feedback function:

For negative feedback loop with forward transfer function G(s) and feedback transfer function H(s), the closed loop transfer function is obtained by

>>sys1=tf(num1, den1); sys2 =tf(num2, den2); sys =feedback(sys1, sys2)     <enter>

For unity feedback system, the closed-loop transfer function is obtained by

>>sys1=tf(num1, den1);  sys = feedback(sys1, 1);   <enter>

Test 3:

1. Find the transfer functions for the following block diagrams:

G1

G2

G1

G2

G1

G2

G3

G1

G2

G3

where

2. Find the closed loop transfer functions of negative feedback systems:

(a) 	and   (b)   and

ASSIGNMENT 3

Time response functions:

The function step(sys) will generate a plot of unit-step response with zero initial conditions where sys is the given system. The time vector is automatically selected when t is not explicitly included in the step command. If the time vector t is supplied at which the response will be computed, the command will be step(sys, t). Test the following set of commands:

>> t=0:.01:100;

>>num=[1]; den=[1 0.2 1];

>>sys=tf(num,den);

>>step(sys, t)  <enter>

also verify that>> step(num, den, t) gives same output.

Similarly, impulse(sys) and impulse(sys, t) will generate the impulse response of a system.

Q.1. Write a MATLAB program to study the step response of a first order system and second order system.

Time Domain analysis of a standard 2nd order LTI system

Simulate the following system using MATLAB.

Q.2. Write a MATLAB program to plot the following:

a)	Rise time (tr)  vs

b)	Peak time (tp)   vs

c)	Overshoot (Mp)   vs

d)	Settling time (ts)   vs

by varying the damping ratio  between 0 to 1 (in case of a), b), and c)) and between 0.2 to 1 (in case of d)), keeping  rad/s,  for a unit step input.

Q.3. Write a MATLAB program to plot the following:

a)	Rise time  vs

b)	Peak time (tp)    vs

c)	Overshoot (Mp)    vs

d)	Settling time (ts)    vs

by varying the natural frequency   between 2 to 8rad/s, keeping   and  for a unit step input.

3. Justify each of the plots of the simulated results from the theoretical point of view.

(Hint for Q1)

Create an M-file with the following set of statements and study the time response of a second order system:

clear all;					%clear all variables

t=0:.01:100;				          % t takes values from 0 to100 at an increment of 0.01

wn=input('Enter natural frequency wn:');	% natural frequency wn as input from keyboard

zeta=input('Enter damping ratio zeta:');	%damping ratio zeta as input from keyboard

num=[wn^2];				           % numerator polynomial: wn2

den=[1   2*zeta*wn   wn^2];		           %denominator polynomial: (s2 + 2ζwns + wn2)

sys=tf(num, den);			          %create transfer function as an object

y=step(sys, t);				          %unit-step response

plot(t,y);				         %plotting of the response

(Hint for Q1)

Create afile with the following set of statements and study the time response of a first order system:

clear;

pack;

clc;

T=input('Enter value of time constant T:');

num=[1];

den=[T 1];

t=0:1:14;

y=step(num,den,t);

plot(t,y);

xlabel('Time in seconds');

ylabel('Output y');

title('Unit-step response of first order system');

ASSIGNMENT 4

The command rlocus generates root loci for given transfer functions. Test the following syntaxes:

rlocus (sys) : computes and plots the root locus of the SISO LTI model sys.

rlocus (sys, k): uses a user-specified vector k of gain values.

rlocus (sys1, sys2, ….): draws the root loci of multiple LTI models on a single plot.

[r, k] = rlocus (sys): returns the matrix r of complex root locations for the gains k.

[k, poles] = rlocfind(sys): returns the feedback gain associated with a particular set of poles on the root locus. It is used for interactive gain selection. This function puts up a cross-hair-cursor on the root locus plot to select a particular pole location. The root locus gain associated with this point is returned in k and the column vector poles contains the closed-loop poles for this gain. To use this command, the root locus must be present in the current figure window.

sgrid (z, w): draw a grid showing two lines where damping ratio is z and a semicircle within which natural frequency is w. Right-click the mouse at the intersecting point of the locus and the damping-ratio-line to know the gain, pole location, and %overshoot for the given value of z.

Test 1: Plot the root locus of the open-loop transfer function

Obtain the value of gain, pole location and %overshoot for a damping ratio 0.6 and natural frequency 6 rad/sec.

Test 2: Create an M-file to plot the root locus of the open loop transfer function

Test rlocfind command in this locus.

ASSIGNMENT 5

Bode Plot:

The command bode generates the Bode frequency-response plots. This function automatically selects the frequency values by placing more points in regions where the frequency response is changing quickly. This range is user-selectable using the command bode (sys, w). Since the Bode diagram has log scale, if we choose to specify the frequencies explicitly, it is desirable to generate the vector w using the logspace function. The logspace(d1,d2) function is used to specify the frequency range that will generate a vector of 50 points logarithimically equally spaced between decades 10d1and 10d2. For example, w = logspace(-1,2) may be used to generate 50 points between 0.1 and 100 rad/s.

When invoked with left-hand arguments,

[mag,phase,w]=bode(sys)

[mag,phase]=bode(sys,w)

return the magnitude and phase of frequency response at the frequencies w.

Verify also the command margin(sys)

Here, sys = tf(num, den), numand den are the row matrices with coefficients of numerator and denominator polynomials of the transfer function.

The magnitude may be converted into decibels using the statement

magdB=20*log10(mag)

Nyquist Plot:

nyquist (sys) plots the real and imaginary parts of the frequency response of an arbitrary LTI models sys.nyquist (sys,w) explicitly specifies the frequency range to be used for the plot. To focus on a particular frequency interval, set w = {wmin, wmax}. When invoked with left-hand arguments

[re,im,w]=nyquist(sys)

[re,im]=nyquist(sys,w)

returns the real and imaginary parts of the frequency response at the frequencies w.

Test 1: Sketch the Bode plot for the transfer functions

(a)

(b)

Test 2: Sketch the Nyquist plot for the transfer functions